# PID Tuning – Frequency Response Method

Presented to Dr. Ahmad othman

Ahmad Ibrahim Kasim Mohammed | 6302

Ahmad El-Zaki Ahmad Hassan | 6303

## 1. Introduction

PID controllers are fundamental in control systems due to their versatility and ease of implementation. Among various tuning methods, the frequency-response approach provides a direct link between time-domain performance and frequency-domain stability measures, such as phase margin and crossover frequency.

This report first outlines the standard frequency-domain PID tuning method. Then, it presents a semi-automated implementation based on second-order system approximations, enabling control over maximum overshoot and settling time through systematic calculations.

## 2. Classical Frequency-Response PID Tuning

### 2.1 Design Specifications

Define time-domain specifications:

- Maximum overshoot: M_p (e.g., 0.05 for 5%)

- Settling time: T_s (in seconds)

These are then converted into frequency-domain targets:

- Damping ratio \zeta based on overshoot:

ζ=-ln(Mp)π2+lnMp2

- Required phase margin \phi_{PM} for a second-order approximation:

ϕPM=tan-12ζ1+4ζ4-2ζ2

- Target open-loop phase at crossover frequency:

∠L(jωc)=-180∘+ϕPM

### 2.2 Crossover Frequency Selection

The crossover frequency \omega_c is chosen where the plant’s phase response matches the above target phase. At that frequency, the magnitude of the open-loop transfer function is forced to 1 by adjusting the proportional gain K_p.

### 2.3 Controller Formulas

Based on the plant characteristics and the selected \omega_c, controller gains are calculated as follows:

- P Controller:

Kp=1|G(jωc)|

- PI Controller:

Cs)=Kp1+1Tis, Ti=10ωc

- PID Controller:

C(s)=Kp1+1Tis+Tds, Ti=10ωc, Td=110ωc

### 2.4 Performance Evaluation

The closed-loop system is:

T(s)=C(s)G(s)1+C(s)G(s)

Performance metrics such as rise time T_r, settling time T_s, overshoot M_p, and steady-state error e_{ss} are computed from the step response. Phase margin and gain margin are evaluated from the loop transfer function.

## 3. Semi-Smart PID Tuning Using Second-Order Approximations

This MATLAB-based method automates tuning and checks performance specifications through a frequency-domain approach, incorporating:

- Explicit overshoot and settling time constraints

- Second-order approximation of the system for performance estimation

- Handling of integrators in the plant

### 3.1 Inputs and Setup

- Specify desired M_p and T_s

- Define the plant G(s) using tf(...)

- Detect and account for integrators by scaling T_i

### 3.2 Frequency Domain Computation

- Calculate \zeta and \phi_{PM} from M_p

- Use interpolation to find the crossover frequency \omega_c where the plant phase matches -180^\circ + \phi_{PM}

- Evaluate plant magnitude at \omega_c to compute controller gains

### 3.3 Controller Loop

For each controller type (P, PI, PID):

- Compute gains using formulas above

- Build open-loop transfer function L(s) = C(s)G(s)

- Evaluate closed-loop T(s) and extract:

  - T_s (settling time)

  - M_p (overshoot)

  - T_r (rise time)

  - e_{ss} (steady-state error)

- Check if specs are met:

  - If yes: plot response and print info

  - If no: discard controller

### 3.4 Visualizations and Results

- Annotated step response for successful controllers

- Bode plot of the plant with target phase margin and frequency

- Automatic rejection of controllers that do not meet both M_p and T_s

- Simulink simulation if a controller is found viable.

## 4. Summary of Workflow

- Define M_p and T_s

- Model G(s)

- Compute \zeta, \phi_{PM}, and target phase

- Interpolate plant phase to find \omega_c

- Compute controller gains

- Simulate step response

- Evaluate and select only valid controllers

- Showcase a quick Simulink simulation if a controller is found viable.

## 5. Advantages of the Semi-Smart Method

- Directly links time-domain specifications to controller design

- Automates gain calculation and performance verification

- Adapts to integrating plants by modifying integral time

- Avoids over-tuning or under-tuning by filtering out unsuitable designs

- Easily extendable to more sophisticated methods or additional constraints

## 6. Conclusion

This report covers both traditional frequency-domain PID tuning for 2nd order systems and extends it in a practical semi-automated method using second-order system approximations for higher order systems. By integrating time-domain specifications into frequency-based controller design, this approach simplifies the tuning process while maintaining robust performance. The MATLAB implementation provides a flexible framework for applying these principles to a wide range of control systems.